3L741series

The machine is designed for high-accuracy grinding of ferromagnetic parts by wheel face. It complies with strict requirements of surface finish, parallelism and accuracy.

Features

- The machine allows to grind flat, convex and concave surfaces.

- The machine is equipped with the system of stable outer speed in the zone of grinding and stable feed per table rotation, providing high accuracy and easy work, the same good quality of finishing on the whole table surface and high productivity.

- High wear resistance and accuracy is provided by roller guideways.

| Specification | 3L741V(А)F10 | 3L741VF10-800 |  |
| --- | --- | --- | --- |
| Workpiece |  |  |  |
| Workpiece dimensions - height - diameter - while working with a small-pole plate - minimal height - minimal diameter | 4…350 50…660 1 10 | 4…350 50…800 1 10 |  |
| Maximum workpiece weight, kg | 300 |  |  |
| Machine |  |  |  |
| Diameter of table working surface, mm | 630 | 800 |  |
| Frequency of spindle rotation, r/min | 1460 |  |  |
| Frequency of table rotation, r/min | 8-32 |  | 6-24 |
| Maximum column travel, mm | 430 |  |  |
| Maximum vertical travel of wheel head, mm | 415 |  |  |
| Rapid cross column travel, mm/min | 120 |  |  |
| Rapid vertical travel of wheel head, mm/min | 200 |  |  |
| Automatic vertical feed of wheel head, mcm | 1; 2; 4; 8; 12; 16; 24; 48 |  |  |
| Main workpiece rotation drive power, kW | 4 |  |  |
| Main wheel head drive power, kW | 11 |  |  |
| Total power of machine electric motors, kW | 18 |  |  |
| Tools |  |  |  |
| Grinding wheel dimensions under GOST 52781-2007, mm | 450х80х203 |  |  |
| Dimensions and weight |  |  |  |
| Dimensions (LxBxH), mm | 2860х2730х2660 |  |  |
| Weight, kg | 5460 | 5740 |  |
| Accuracy parameters for test piece |  |  |  |
| Working accuracy: - flatness, mcm - parallelism, mcm - surface roughness, mcm | 4 4 0,25 | 6 6 0,5 |  |